# Getting Started with Simulink CIGRE Build Process

This guide will help you set up and configure the CIGRE build process in Simulink. Follow these steps to get started.

# Prerequisites

- Toolchain

  - Visual Studio for C compiler

    - 2017

    - 2019

    - 2022

  - MinGW

- MATLAB/Simulink ver. R2020a or later

  - Simulink Coder

  - Embedded Coder

- CIGRE mltbx

# Toolchain Setup

### Visual Studio

Install a supported version of Visual Studio, along with C/C++ toolchains.

### MinGW

If not already set up, install MinGW support package through MATLAB:

You may need to restart MATLAB.

Run

compilers = mex.getCompilerConfigurations('C', 'Installed');
minGW = compilers(ismember({compilers(:).Name}', 'MinGW64 Compiler (C)'))

Add the path specified for the LinkerName to the path:

Open System Environment Variables:

Go to “Advanced -> Environment Variables”

Add the path, e.g. “C:\ProgramData\MATLAB\SupportPackages\R2023b\3P.instrset\mingw_w64.instrset\bin\” to the path”.

Restart MATLAB.

In MATLAB, run

!gcc

If you get the following error, it should be configured correctly:

# Installation

- Install the CIGRE toolbox by running CIGRE.mltbx inside your MATLAB environment.

- Create the required CIGRE DLL toolchains by running:

cigre.install(“VSVersion”, <<version>>)

where <<version>> matches an installed version of Visual Studio.

This will create both 32- and 64-bit CIGRE DLL toolchains and register them with Simulink.

- Check the installation of the model advisor checks by:

cigre.test.advisorChecks

# Model Configuration

- Open the model configuration.

- Navigate to Hardware Target and choose whether you want to generate a 32- or 64-bit DLL.

- Navigate to Code Generation and set the System target file to ‘cigre.tlc’

- Select the appropriate CIGRE DLL toolchain

# Model Checker

To check if a model is compatible with the CIGRE DLL code generation run checks on the model.

cigre.checkModel(<<modelName>>)

For details press the Summary Report link.

To run the model advisor interactively, open the model advisor from the modelling pane.

Select the CIGRE checks under the By Product folder:

And run them.

# Building the Model

- The model is build by calling

cigre.buildDLL(<<modelName>>)

where <<modelName>> is the name of the top level model that you wish to build. This will create a DLL called <<modelName>>_CIGRE.dll, along with a header <<modelName>>_CIGRE.h that describes the DLL interface.

# Known Limitation

- The top-level model cannot contain virtual buses.

- Logical/Boolean data in Simulink is converted to int32 in CIGRE.

- Compatibility mode for triggered subsystem initialisation is not supported due to how the initialisation code is generated:

- (u)int64 is not generally supported as this requires support for multi-word typedefs

- Must use fixed time-step

- Model must have at least one input